Standard Actions, Callouts, and Responses for Pilot Flying,

and Pilot Not Flying

| Action | PF Call/Response | PNF Call/Response |
| --- | --- | --- |
| Start Button Depress | “Time” | “Time” “HYD Pressure Up, Pump Light Out, Oil Press Light Out, Fuel Press Light Out” |
| Starting Engine Stabilized | “NG and T5 Stabilized, Starter Disengaged, Bleed (Eng #) to High” “Clear (Eng #)” | “Bleed (Eng #) to High Clear (Eng #)” |

Figure 6-2.  Engine Starting Standard Callouts

| Action | PF Call/Response | PNF Call/Response |
| --- | --- | --- |
| Power Lever Advance for Takeoff | “Set Power” | “Spoilers Retracted, Autofeather Armed, Power Set” |
| 50 KIAS (Indicators and Systems Normal) |  | “50 Knots” |
| Airspeed at V1 |  | “V1, Rotate” |
| Positive Rate of Climb | “Gear Up, Bleeds 1 and 4 Low” | “Positive Rate” “Gear Selected Up, Bleeds 1 and 4 Low” |
| 400’ AGL | “Accelerating” | “400 Feet, Looking for (Flap Retraction Airspeed)” |
| Flap Retraction Airspeed (V2 for Flaps Zero) | “Airspeed (State Speed) Select Flaps Zero” | “Flaps Selected Zero” |
| Third Segment Climb | “Autofeather Off, Set Climb Power, After Takeoff Check” “My Power” | “Your Power” (After Deslecting Autofeather and Setting Power) “After Takeoff Check Complete” (After Competing CL) |

Figure 6-3.  Takeoff Standard Callouts

| Airspeed Deviates +5/-0 KIAS From V2 or V2+10 Speed | “Increasing/Decreasing Airspeed” | “Airspeed __ Knots Slow/Fast” |
| --- | --- | --- |

Figure 6-4.  Takeoff Deviations Standard Callouts

| Action | PF Call/Response | PNF Call/Response |
| --- | --- | --- |
| Abnormal or Emergency Condition Prior to V1 (Identified by PNF) | “Aborting” | “Abort” (State Problem) |
| PF Elects to Abort Prior to V1 | “Aborting” (State Problem, Time Permitting) | “Roger” |
| Engine Malfunction at or after V1 (Sequence depends on where failure occurs) | “Gear Up” (Climb at V2) “Confirmed” | “Engine Failure on Number 1, Max Power Set, Positive Rate” “Gear Selected Up, Prop did not Feather” “Number 1 Condition Lever Identified” “Fuel Shut-Off” |
| 400’ AGL | “Accelerating” | “Above 400 Feet, Looking for (Airspeed)” |
| Flap Retraction Airspeed | “Airspeed (State Speed) Select Flaps Zero” | “Flaps Selected Zero” |
| Third Segment Climb | “Continue the Emergency Procedure” “Confirmed” “Roger” “Confirmed” “Complete the Emergency Procedure Checklist” | “Number 1 Power Lever Identified” “Flight Idle, Number 1 Condition Lever is at Fuel Shut-Off” “Number 1 Pull Fuel Off Handle Identified” “Pulled, No Indication of Fire” |

Figure 6-5.  Takeoff Emergencies Standard Callouts

| Action | PF Call/Response | PNF Call/Response |
| --- | --- | --- |
| Transition Altitude | “29.92” Set Left | “29.92” Set Right |
| At Cruise Speed | “Set Cruise Power, Cruise Check” “My Power” | “Your Power” (After Setting Power) “Cruise Check Complete (After Completing CL) |
| 10,000 or Below | “Descent Arrival Check” | “Descent Arrival Check Complete” (After Completing CL) |
| New Assigned Altitude |  | (Altitude) Set and Armed |
| Climb or Descent | “Climbing/Descending to (Assigned Altitude)” | “Roger”, (Report Altitude Change to ATC) |
| Within 1000 Feet of Assigned Altitude | “Passing (Altitude) for (Assigned Altitude)” | “1000 to Go” |

Figure 6-6.  Climb, Cruise, and Descent Standard Callouts

| Action | PF Call/Response | PNF Call/Response |
| --- | --- | --- |
| Initial Course/Localizer Movement | “Course/Localizer Alive” | “Roger” |
| Course/Localizer Capture | “Course/Localizer Captured” | “Roger” |
| Initial Glideslope Movement (Precision Approach) | “Glideslope Alive” | “Roger” |
| Glideslope Capture (Precision Approach) | “Glideslope Captured” | “Roger” |
| Approaching the FAF | “Airspeed Below 147, Select Flaps 15, Select Gear Down, Before Landing Check” | “Flaps Selected 15, Gear Selected Down”, “Before Landing Check Complete” (After Completing CL) |
| Prior to the FAF (Stabilized Approach) | “Airspeed Below 124, Select Flaps 25” “Condition Lever Max RPM, Landing Check” | “Flaps Selected 25” “Gear Rechecked Down, Flaps are 25, Condition Levers Max RPM, Landing Check Complete” |
| FAF (Non-Precision Approach) | “Time” | “Time Started” |
| 100’ Prior to DH/MDA | “Roger, Continuing” | “100 feet Above DH/MDA” |

Figure 6-7.  Instrument Approach Standard Callouts

| # Action | PF Call/Response | PNF Call/Response |
| --- | --- | --- |
| DH, Rwy Environment Insight | “Continuing” (Or other intentions) | “DH, Approach Lights (Or other features identifiable with RWY environment) In Sight” (Position) |
| RWY In sight | “RWY Insight, Landing” | “RWY In sight” (Position) |
| DH, RWY Environment Not in Sight | “Missed Approach” | “Missed Approach” |

Figure 6-8.  Precision Approach Standard Call-outs

| Action | PF Call/Response | PNF Call/Response |
| --- | --- | --- |
| MDA | “Continuing” | “Minimums” (State time/distance to Go) |
| RWY Environment Insight | “Continuing” (Or other intentions) | “Approach Lights (Or other features identifiable with RWY environment) In sight” (Position) |
| RWY Insight | “RWY Insight, Landing” | “RWY In sight (Position)” |
| RWY Environment Not In sight at MAP | “Missed Approach” | “Missed Approach” |

Figure 6-9.  Non-Precision Approach Standard Callouts

| Action | PF Call/Response | PNF Call/Response |
| --- | --- | --- |
|  One Dot Off Glideslope | “Correcting (Up/Down)” | “One Dot (High/Low) and Increasing/Decreasing” |
|  One Dot Off Localizer/VOR | “Correcting (Left/Right)” | “One Dot (Left/Right) And Increasing/Decreasing” |
|  5 on NDB Approach | “Correcting (Left/Right)” | “__ (Left/Right) and Increasing/Decreasing” |
|  10 Knots from Approach Speed | “Increasing/Decreasing Airspeed” | “Airspeed __ Knots Slow/Fast” |
| Vref + 10/-0 Knots | “Increasing/Decreasing Airspeed” | “Vref  __ Knots” |
| Rate of Descent Exceeds 1000 Feet per Minute | “Reducing Sink Rate” | “Sink Rate (Amount) Increasing/Holding” |

Figure 6-10.  Approach Deviations Standard Callouts

| Action | PF Call/Response | PNF Call/Response |
| --- | --- | --- |
| Go-Around (Flaps 45) | (Push Power Levers Up, Rotate to 8-12 Pitch Attitude) “Set Maximum Power, Flaps 25” | “Maximum Power Set, Flaps Selected 25” |
| Descent Stopped | “Gear Up” | “Positive Rate” “Gear Selected Up, Looking for (Flaps 15 V2 Airspeed)” |
| Flap Retraction Airspeed (25 to 15) | “Airspeed (State Speed) Select Flaps 15” | “Flaps Selected 15, Climb at (V2+10 Airspeed)” |
| Above 400 Feet AGL | “Accelerating” | “Above 400 Feet, Looking for (Airspeed)” |
| Flap Retraction Airspeed (15 to Zero) | “Airspeed (State Speed) Select Flaps Zero” “Go-Around Checklist” | “Flaps Selected Zero” |

Figure 6-11.  Go-Around/Missed Approach Standard Callouts

| Action | PF Call/Response | PNF Call/Response |
| --- | --- | --- |
| Bank Angle Exceeds 30 | “Correcting” | “Bank Angle” |
| Airspeed Deviates  10 KIAS | “Increasing/Decreasing Airspeed” | “Airspeed __ Knots Slow/Fast” |
| Altitude Deviates  100 Feet | “Increasing/Decreasing Altitude” | “Altitude __ Feet Low/High” |
| Heading Deviates  10 | “Correcting Left/Right” | “Heading __  Left/Right” |

Figure 6-12.  All Phases of Flight Standard Callouts

| Action | PF Call/Response | PNF Call/Response |
| --- | --- | --- |
| Airspeed Decelerating through 40 KIAS | “Lock the Controls” | “40 Knots” “Controls are Locked” (After taking control of the yoke and engaging the control lock) |
| Crosswind Landing | “Your Yoke, My Tiller” | “I Have the Yoke” |
| Right Seat Pilot Taxiing the Aircraft | “Nose Wheel Steering Off” | “Nose Wheel Steering Off” |

Figure 6-13.  Landing Roll Standard Callouts